Primary

Fuel Pump

*Note: Standard FP output does not support PWM, either map an existing unused PWM output or rewire and relay 12v to FP and use standard

**Programmable PWM is best solution, FP ECU behaviour: http://www.supraforums.com/forum/showthread.php?649900-Anyone-try-PWM-control-of-the-fuel-pumps

12V switched and ground

Sensor 5V

Sensor Ground

Cam & Crank

G1 = intake cam		Custom Harness = green

NE = crank		C H = blue

G- = ground		C H = grey

*Refer to Nathaninwa post

Injectors

*Measure resistance first before connecting in case of low impedance

IAC

*Refer to 2jz-ge engine diagnostics.pdf to see exact signals needed

TPS

IAT

O2

Map

Cranking

Other

** M-Rel jumped to IGSW (Must keep ecu on for a few seconds before cranking)

Secondary

Speed

*Digital Frequency 1

CEL

ACIS (long-shortrunners)

Coolant Temp (CLT)

Knock

Left to do:

- cnk and cmp split common ground and wire up

- ignition coils output wires

- usb wires